GLA-1000 Specifications

The specifications apply when the GLA-1000 is powered on for at least 30 minutes under +20°C~+30°C.

|  | GLA-1016 | GLA-1032 | GLA-1132 |
| --- | --- | --- | --- |
| Channels | 16 | 32 | 32 |
| MEMORY TOTAL/PER CHANNEL | 4Mbits/256kbits | 4Mbits/128kbits | 32Mbits/1Mbits |
| INTERFACE | USB 2.0 (1.1) |  |  |
| OPERATING SYSTEM | Win XP/Win Vista /Win 7 |  |  |
| TIMING ANALYSIS | Maximum 200MHz |  |  |
| STATE ANALYSIS | 0.001Hz ~ 100MHz |  |  |
| TRIGGER |  |  |  |
| Channel | 16 | 32 | 32 |
| Condition | Edge/Pattern |  |  |
| Pre/Post trigger | 0%~100% |  |  |
| Level | 1 |  |  |
| Threshold | +6V ~ -6V |  |  |
| Accuracy | ±93mV |  |  |
| Delay | Time delay, Clock event delay |  |  |
| Count | 1 ~ 65535 |  |  |
| Page | Maximum 8191 |  |  |
| INPUT |  |  |  |
| Maximum Input Voltage | ±30V |  |  |
| Impedance | 500kΩ/10pF |  |  |
| Data Skew | <1.5ns |  |  |
| ENABLE QUALIFIER |  |  |  |
| Channel | 16 | 32 | 32 |
| Enable Condition | Don't Care ,High ,Low |  |  |
| Enable Delay | 1~65535ms |  |  |
| DATA |  |  |  |
| Compression Channels | 16 | 24 | 24 |
| Compression Ratio | Max.255 |  |  |
| Signal Statistics | Postive/Negative, Full cycle within length condition |  |  |
| Protocol Analyzer | I2C, RS-232C ,SPI |  |  |
| CURRENT CONSUMPTION |  |  |  |
| Static | Max. 200mA |  |  |
| Working | Max. 400mA |  |  |
| POWER DISSIPATION |  |  |  |
| Static | Max. 1W |  |  |
| Working | Max. 2W |  |  |
| POWER |  |  |  |
| Interface | USB |  |  |
| Working Voltage | 4.5V~5.5V |  |  |
| DIMENSIONS & WEIGHT |  |  |  |
|  | 102(D) x 39 (H) x 137(W) mm ; Approx. 340g |  |  |